| Document No: | [Subject] | Issue: | [Status] | Effective date: | [Publish Date] |
| --- | --- | --- | --- | --- | --- |

| Document Title: |  |  |
| --- | --- | --- |
| [Title] |  |  |
| Summary / Scope: |  |  |
| Reason for Issue / Nature of Change: |  |  |
| Issue; Date; Details – reason for issue / nature of change. 01. 02. 03. |
| AUTHORISATION. Name:; Job title:; Signed:. Author:; Click to enter text. Reviewer:; Click to enter text. Approver / Owner:; Click to enter text. Authorisation details required irrespective of document approval process applied.; Electronic signatures required only where SharePoint approval workflows are not being used.. |

Copyright Statement

© 2026 Renishaw plc

This document is the property of Renishaw plc and the information contained herein is confidential.  The contents of the document must not be reproduced or disclosed wholly or in part, or used for purposes other than that for which it has been supplied, without the prior written permission of Renishaw plc, or, if it has been furnished under a contract with another party, as expressly authorised under that contract.

Table of Contents

1	Introduction	3

2	Component Calculations	3

2.1	Triangle & PWM Wave Oscillator	3

2.2	Sin Wave Oscillator	5

2.3	Output Circuit	7

# Introduction

# Component Calculations

## Triangle & PWM Wave Oscillator

The frequency (f) of the triangle & PWM oscillator is found from the sum of charge and discharge times of a capacitance between upper and lower threshold voltages (VU and VL) when a constant drive voltage (Vd) is applied. PL1 is to be fitted with a 100kΩ variable resistance. The frequency is calculated by

f=Vd2RTCTVT

Where:

CT=SW1=0 SW1=1 C14C14+C13

RT=PL1+R16

VT=2∙R23R24Vd

R23 and R24 define the amplitude of the triangle and contribute to the frequency of the oscillator. The signal needs to be brought away from the rails to prevent distortion, however larger amplitudes allow lower frequencies – and so a target amplitude of 10V was defined. Values of 36kΩ and 39kΩ were chosen to achieve this value.

V=3639∙11V≈10V

C13 and C14 were chosen to be a factor of 10 apart to allow the oscillator to reach a greater range of frequencies while still allowing some crossover to ensure it can cover the entire range. When using a 100k variable resistor, values of 10nF and 100nF are needed to place the available frequency range correctly as well as a series resistance (R16) of 7.5kΩ for an offset shift.

To create the PWM signal, a reference voltage is needed within the bounds of the triangle signal. R18 and R20 were set at 22kΩ combined with RV2 of 100kΩ, to limit the voltage threshold range down to a useful level.

VL=IRL=12-(-12)10000+2∙2200∙2200=3.67

12V-3.67V=8.33V

R19 was added as a 10kΩ resistance to protect the op amp IC4 from ESD from the potentiometer. Similarly, D6 is in place to protect the inputs of IC3. A 1MΩ resistance (R21) was added across IC4 to add hysteresis in case the potentiometer is noisy. R22 and R30 were selected to reduce the output level of the comparator from the rails to the target amplitude. C24 was put in place as a small capacitance (22pF) to remove the harsh transient spikes that arose from the PWM gain.

V=3643∙12V≈10V

## Sin Wave Oscillator

The frequency (f) of the sin wave oscillator is determined by the RC network between the output and non-inverting input of the centre op amp (IC5). The series and parallel networks must remain symmetric to amplify a specific frequency at a time.

f=12π∙RTCT

Where:

CT=SW1=0 SW1=1 C19== C20(C19+C21)==(C20+C16)

RT=PL2+R17==PL4+R26

PL2 and PL4 are to be connected to variable resistances of 100kΩ. C19 and C20 are 10nF and C16 and C21 are 100nF to match the triangle oscillator. R17 and R26 are chosen to be 4.7kΩ to most closely match the frequency curve of the other oscillator, allowing access to the desired range with sufficient crossover between capacitor settings.

The gain feedback control relies on JFET TR1 and surrounding components to regulate the amplitude across frequencies. The JFET acts as a voltage-controlled resistor which sums with R28 to make a gain compared to R25 and PL3. The ratio of (RV4+R11):(TR1+R8) needs to always be exactly (or just over) 2:1. If R28 is set at 10kΩ then R25 can be 15kΩ such that when combined with PL3 ranging over 10kΩ, the total can range from 15k to 25k adjustable.

The time constant created by R29 and C23 should be at least 100x the period of the lowest frequency available.

R29∙C23≫120Hz=0.05

Choosing R29 to be 100kΩ leaves C23 needing to be greater than 50µF and so 100µF is chosen for safety. The small capacitor C22 is there to filter out high frequency noise and was chosen as 22pF.

## Output Circuit

R1, R2 and R5 are pulling the input pins of the mux to ground for a more stable logic input. 10k was selected for this purpose. R3 and R4 are put in place to protect the IC1 inputs from ESD from SW1.

For volume control, the signal needs to be attenuated from +/-10V down to 1V rms. The combination of RV1 and R11 mimics logarithmic control to match the exponential human hearing while the op amp in IC2 uses R12 and R13 to attenuate the resultant signal.

V=22150∙10≈1.4V

Vrms=1.4V2=1V

There is a series capacitance at the output to remove any DC bias from the signal. This creates a filtering effect. Combined with the load of the output and R15.