| Document No: | [Subject] | Issue: | [Status] | Effective date: | [Publish Date] |
| --- | --- | --- | --- | --- | --- |

| Document Title: |  |  |
| --- | --- | --- |
| DCO User Manual |  |  |
| Summary / Scope: |  |  |
| This document describes the I/O and functionality of the Digitally Controlled Oscillator (DCO) board. It will show the relevant pinouts and connections required to interface with the Xmega Xplained development board as well as the calculations and processes required to get the basics of the board operating. |  |  |
| Reason for Issue / Nature of Change: |  |  |
| Issue; Date; Details – reason for issue / nature of change. 01. 02. 03. |
| AUTHORISATION. Name:; Job title:; Signed:. Author:; Benjamin Bolton; Click to enter text. Reviewer:; Click to enter text. Approver / Owner:; Click to enter text. Authorisation details required irrespective of document approval process applied.; Electronic signatures required only where SharePoint approval workflows are not being used.. |

Copyright Statement

© 2026 Renishaw plc

This document is the property of Renishaw plc and the information contained herein is confidential.  The contents of the document must not be reproduced or disclosed wholly or in part, or used for purposes other than that for which it has been supplied, without the prior written permission of Renishaw plc, or, if it has been furnished under a contract with another party, as expressly authorised under that contract.

Table of Contents

1	System Architecture	3

1.1	Module Overview	3

1.2	Analogue Outputs	4

2	Theory of Operation	4

2.1	Triangle and PWM Signals	4

2.2	Sin Wave Signal	5

2.3	Operational Overview	6

3	Interfacing Guide	6

3.1	Capacitance Selection	6

3.2	Digipot Interface	7

3.3	Frequency Measuring	9

3.4	Rotary Encoder Interface	9

3.5	Multiplexer Interface	9

3.6	Extra functionality	10

3.7	Default features	10

4	Pinout & Pin Descriptions	10

4.1	Capacitor Selection Pins	10

4.2	Digipot Pins	11

4.3	Frequency Measurement Pins	11

4.4	Rotary Encoder Pins	11

4.5	Multiplexer Pins	12

4.6	LCD Pins	12

4.7	Switch Pins	12

4.8	Extra Pins	13

5	Test Points	13

6	Appendix	14

6.1	Appendix A	14

6.2	Appendix B	15

6.3	Appendix C	17

6.4	Appendix D	18

6.5	Appendix E	19

# System Architecture

## Module Overview

Figure 1 - System Block Diagram

The core of the module revolves around the oscillator circuitry which is controlled digitally by the Xmega. User inputs consist of a 24-detent rotary encoder; a volume control potentiometer; and the 4 existing buttons present on the Xmega development board. The module has three individual BNC outputs paired with corresponding LEDs for signal visualisation, as well as one main audio jack for signal outs. The module also has full control of the LCD screen as well as some spare LEDs for any extra functionality required.

## Analogue Outputs

The three analogue outputs are triangle, PWM and sin waves, each operable within a frequency range of roughly 35 - 3500Hz. The three signals are each directly accessible via the BNC output connectors and have been designed to operate at ±10V.

The wave signals also feed into a multiplexer where one can be selected at a time to be the audio output via a 3.5mm jack. This stage is designed to attenuate the chosen signal down to a maximum amplitude of 1.4V (1VRMS) and remove any DC bias from the signal for compatibility with further audio drivers. The volume potentiometer allows further control of the signal’s amplitude from the maximum of 1.4V down to 0V.

# Theory of Operation

Within the module there are two oscillator cores that are controlled separately, the first generates both the triangle and the PWM signal while the second generates the sin wave. Each oscillator has its frequency controlled by digital potentiometers which range from 0 – 97.5kΩ [Table 4 in datasheet], as well as two selections of capacitance (10nF & 110nF) which creates two “bands” of available frequencies with some overlap between them.

## Triangle and PWM Signals

A triangle wave is generated by charging and discharging a capacitor continuously with a constant current. By varying the current flow to the capacitor, the rate at which it charges and discharges is affected directly. As the capacitance is always going between two known constant voltage levels – the rate of charge is proportional to the period of the signal. The derivation is shown in Appendix A and the symbolic equation comes out to be

fTRI=14HRC

Where H is defined as the ratio of resistors around the comparator. In the case of this module these two resistors are 36kΩ and 39kΩ. There is also a series resistance of 4300Ω in the feedback and so the final equation with 4∙H=4∙3639≈3.7 is

fTRI=13.7∙4300+R∙C

The PWM signal is generated from comparing the triangle wave output to a controlled voltage level. Raising and lowering the voltage will alter the duty cycle of the signal. In theory the equation for the Duty cycle, Dcyc is

Dcyc=10V-Vref20

Instructions on setting Vref are detailed in section 3.2.

As the DC bias is removed from the main output signal – at this stage the PWM signal’s duty cycles will appear equal with opposite values e.g. 25% and 75% will sound the same. The BNC connections are not affected by this.

## Sin Wave Signal

The sin wave is generated using a Wien bridge oscillator. It uses a network of resistors and capacitors in an arrangement like Figure 2 which acts as a band pass filter to select a calculated frequency.

R1

Vin

C1

R2

C2

Vout

GND

GND

Figure 2 – Wien bridge network

According to the reactance of a capacitance

χC=12πfC

At low frequencies, the reactance of the capacitors is very large and so C1 blocks the input and the output voltage is 0V. At high frequencies the reactance approaches zero and so C2 shorts to ground also pulling the output to 0V. Assuming that the values of the resistors and capacitors are the same in each branch, somewhere in the middle of the frequency range must lie a frequency point which is attenuated the least. Feedback amplification is built around the filter network such that random noise signals will start the oscillation, and the selected frequency will dominate the resultant signal.

The derivation of the frequency equation is shown in Appendix B and comes out to be

f=12πR1R2C1C2

In this module, the two capacitor values must always be equal, and it is assumed that the resistances are to be set to equal values too. However, this can be adjusted to increase the bandwidth. With these assumptions and the addition of a 1200Ω series bias resistance, the working frequency equation is

f=12π∙1200+R∙C

The Wien bridge oscillator also requires fine gain control. Theoretically there must be a minimum gain of 3 for the circuit to continue oscillating as shown in Appendix B - but in practice it must be slightly more than 3 due to signal losses. Too much gain will quickly start to distort the signal and so careful control of finding the minimum value possible for the circuit to remain oscillating is required. This additional gain is also controlled by a separate resistance network where on this board the gain, β is

β≈1+R+4.7k10k

## Operational Overview

The board needs to be supplied with +17-24V to function. This can be done either via the DC plug or the terminal block located next to it. Only one of these options should be used at a time.

As discussed above, the frequencies of the oscillator cores both rely on a variable resistance value. This module includes digital potentiometers in place of all resistance values R in described equations. Each oscillator also has two capacitor modes, allowing the user to flip between a 10nF and 110nF capacitances. This essentially creates two frequency bands, a higher and lower, that can both extend the full range of the varying resistance.

From a target frequency, a frequency band can be selected and then a resistance value estimated using the above equations. The outputs of the oscillators are clamped and fed back into the Xmega so the actual frequency can be measured. Using this, the resistance value can be updated in real time until the measured frequency matches the target frequency.

Difficulty arises when choosing a capacitance around the crossover point of the frequency bands.

Figure 3 shows the idealistic relationship between the resistance of the Digipots and resultant frequency of the sin wave Wien oscillator. Navigating the overlap between the frequency bands is a challenge and care should be taken to avoid repeated flipping of the capacitance. It should be noted that the higher band (C=10nF) is generally preferred as there is more resolution per step change within this period, but more complicated switching mechanics involving hysteresis is advised. Appendix E shows more accurate estimations of frequency on log graphs.

Figure 3 - Graph showing crossover region of frequency bands

# Interfacing Guide

## Capacitance Selection

By default, all oscillators are in the upper frequency (C=10nF). However, each oscillator has extra 100nF capacitors through an analogue switch in parallel with the 10nF. Providing the pins for capacitor selection with a logic HIGH will enable the additional capacitance and bring the total to 110nF. Both oscillator cores have their own capacitor selection pin so they can be controlled individually.

## Digipot Interface

The digipots communicate with the Xmega with an SPI bus. There are TX, RX, CLK, SYNC and RDY pins assigned. On this module, the digipots are configured in daisy chain mode. This means that the Xmega has its TX pin connected to the Data In of one Digipot, and then the Data Out of the first pot is connected to the Data In of the next. Therefore, in order to update the value of one Digipot – you will need to update the values of all Digipots simultaneously. The order of the daisy chain is described in Figure 4 and Table 1.

Figure 4 - Digipot daisy chain

Because the signals are propagated through each of the Digipots, the commands should be sent in reverse order (RV6, RV5, RV4 etc…) such that they each arrive at the intended Digipot once the full command string is complete.

Table 1 - Digipot descriptions

| Digipot | Max Value (RAB) | Description |
| --- | --- | --- |
| RV2 | 20kΩ | Gain control resistance of Wien Oscillator |
| RV3 | 100kΩ | The high pass filter control resistance of the Wien oscillator |
| RV4 | 100kΩ | The low pass filter control resistance of the Wien oscillator |
| RV5 | 100kΩ | The frequency control resistance of the triangle oscillator |
| RV6 | 100kΩ | The resistance controlling the mark-space ratio of the PWM signal |

The SYNC and RDY pins on all Digipots are tied together respectively and connected to one pin each of the Xmega. As detailed in the Digipots’ datasheet, the SYNC pin must be held low while the data is clocked through all 5 Digipots and then released only after it has all been sent. As the RDY outputs are also tied together, reading the level of the RDY pin will give limited information about all of the pots simultaneously. The use of the RDY pin is optional and only required to accommodate tighter timings allowing for an increased rate of commands sent to the Digipots.

Another important note is the initialisation of the Digipots. A control bit must be written to the control register in order for the subsequent data commands to actually update the wiper position. Without this, the Digipots will be locked to 50% of their max value. Optionally, in daisy chain mode, the Digipots’ data out pins can be put into high impedance mode to reduce power consumption.

The datasheet [X:/DataSheetsAndImages/PDFs/Resistors/AD5293-AnalogDevice.pdf] includes additional information and detailed instructions regarding commands to send and setup instructions.

Digipots RV2-5 are wired such that the functional resistances are between the A and W terminals of each Digipot. The equation for resistance between the A and W terminals is

RAW=1024-D1024×RAB

Where D is the decimal equivalent of the binary code sent to the Digipot register and RAB is the maximum resistance of the Digipot.

Digipot RV6 is used for the mark-space ratio for the PWM signal by generating a reference voltage Vref between ±12V. There are some 22kΩ offset resistors between the rails as shown in Figure 5.

22kΩ

22kΩ

100kΩ

+12V

-12V

Figure 5 - PWM Reference Divider

The equation for the reference voltage is therefore

Vref=24×22k+RWB22k+22k+RAB-12

RWB=D1024×RAB

∴Vref=24×22k+D1024∙100k144k-12

Alternatively, D1024 can be thought of as just the ratio between the min and max values. E.g. A symmetrical square wave can be generated by writing D=512.

The Digipot communication lines are connected to the default USARTC0 bus lines on the Xmega for simple setup and transmission/receiving of data – refer to the Xmega specific guides for this setup.

## Frequency Measuring

The outputs of the oscillator cores are each directly fed back into the Xmega via some clamping diodes into two separate pins. The diodes ensure the ±10V signals are truncated down to a range of 0 - 3.3V to protect the Xmega inputs as shown in Figure 6. This also ensures use of the largest dVdt section of the sin wave for better sensing on the input. The PWM signal is also clamped down to the same range but as it is already rectangular pulses, the shape is not changed.

Figure 6 - Clamping Signal Plot

The method of measuring frequency is to be determined by the user, but it may be worthwhile looking into the Xmega event system for a more refined approach.

## Rotary Encoder Interface

The board includes a mechanical rotary encoder intended to be used as the primary user interface and input into the system. It includes two terminals for a quadrature signal and one for a push switch. While the Xmega comes packaged with quadrature decoding hardware, the pin setup from the encoder is not laid out to make use of it and such a decoding system must be written from scratch. The push button on the encoder can be treated as a standard active LOW switch.

If the filtering capacitors are fitted on the board, then the three inputs should be naturally debounced, otherwise bounce filtering should be handled in software.

## Multiplexer Interface

The multiplexer takes a 2-bit binary code to select between the 3 signal waveforms. The fourth combination output is tied to GND allowing quick disabling of the main output or silent periods without disabling any ICs. The signal selection codes are shown in Table 2.

Table 2 – Multiplexer selection codes

| Code | MUX_SEL_1 | MUX_SEL_0 | Output |
| --- | --- | --- | --- |
| 00 | 0 | 0 | PWM |
| 01 | 0 | 1 | TRIANGLE |
| 10 | 1 | 0 | SIN |
| 11 | 1 | 1 | GND |

## Extra functionality

There are a few extra features on the board including a spare analogue switch and some extra LEDs that can be used or ignored depending on specific design requirements. The switch is open on both sides and routed to empty pads to connect to any external circuitry. The LEDs are active LOW. There is also a spare op amp, routed in the negative feedback configuration with empty pads on inputs and outputs and space for resistors. The connection points are routed according to Figure 7.

Figure 7 - Spare op amp configuration

## Default features

All standard features of the Xmega Xplained board have been kept intact allowing full functionality for existing hardware. This includes the LCD, onboard switches and LEDs, flash memory and all other sensors included on the board, allowing for some interesting possibilities such as temperature-controlled frequency for example. Refer to the Xmega manuals for pinouts and interfacing guides with these standard features.

# Pinout & Pin Descriptions

For ease of finding relevant pin information, the pinouts have been organised in terms of their relevant functionality rather than position on the board.

## Capacitor Selection Pins

| Label | Xmega I/O | Xmega Connector | Signal type | Description |
| --- | --- | --- | --- | --- |
| C_SEL_SIN | PB2 | J2.3 | GPIO – OUT | A digital signal to control capacitance selection for Sin wave. LOW for 10nF, HIGH for 110nF |
| C_SEL_PWM | PA4 | J2.5 | GPIO – OUT | A digital signal to control capacitance selection for Triangle and PWM signals. LOW for 10nF, HIGH for 110nF |

## Digipot Pins

Table 3 - Xmega Digipot pin list

| Label | Xmega I/O | Xmega Connector | Signal type | Description |
| --- | --- | --- | --- | --- |
| DIGI_CS | PC0 | J1.1 | GPIO – OUT | The CHIP SELECT or SYNC pin used to initialise and confirm a data transmission with the Digipots. Must be pulled LOW during transmission and released when completed |
| DIGI_CLK | PC1 | J1.2 | USART – OUT | The CLOCK pulse output for the SPI connection to the Digipots |
| DIGI_MISO | PC2 | J1.3 | USART – IN | The MASTER IN pin for the SPI connection to the Digipots |
| DIGI_MOSI | PC3 | J1.4 | USART – OUT | The MASTER OUT pin for the SPI connection to the Digipots |
| DIGI_RST | PC4 | J1.5 | GPIO – OUT | The RESET pin for the Digipots. Leave pin as input to automatically tie the Digipot pins HIGH. Otherwise, a hard reset is performed on a LOW to HIGH transition |
| DIGI_RDY | PC6 | J1.7 | GPIO – IN | The READY pin from the Digipots. Active HIGH signal identifies the completion of a write or read operation to/from the Digipots’ registers |

## Frequency Measurement Pins

Table 4 - Xmega Frequency pin list

| Label | Xmega I/O | Xmega Connector | Signal type | Description |
| --- | --- | --- | --- | --- |
| F_SIN | PE0 | J4.1 | GPIO – IN | A clamped version of the sin wave signal directly from the oscillator core ready for edge sensing |
| F_PWM | PA7 | J2.8 | GPIO – IN | A clamped version of the PWM signal directly from the oscillator core ready for edge sensing |

## Rotary Encoder Pins

Table 5 - Xmega Encoder pin list

| Label | Xmega I/O | Xmega Connector | Signal type | Description |
| --- | --- | --- | --- | --- |
| ENC_A | PE1 | J4.2 | GPIO – IN | Quadrature signal A from the rotary encoder (leading for CW) |
| ENC_B | PE3 | J4.4 | GPIO – IN | Quadrature signal B from the rotary encoder (lagging for CW) |
| ENC_SW | PE2 | J4.3 | GPIO – IN | Active LOW push button switch on the rotary encoder |

## Multiplexer Pins

Table 6 = Xmega MUX pin list

| Label | Xmega I/O | Xmega Connector | Signal type | Description |
| --- | --- | --- | --- | --- |
| MUX_SEL_0 | PB3 | J2.4 | GPIO – OUT | Bit 0 of the binary selection code for the signal multiplexer |
| MUX_SEL_1 | PB1 | J2.2 | GPIO – OUT | Bit 1 of the binary selection code for the signal multiplexer |

Refer to Table 2 for MUX codes.

## LCD Pins

Table 7 - Xmega LCD pin list

| Label | Xmega I/O | Xmega Connector | Signal type | Description |
| --- | --- | --- | --- | --- |
| LCD_CS | PF3 | - | GPIO – OUT | The CHIP SELECT or SYNC pin used to initialise and confirm a data transmission with the LCD. Must be pulled LOW during transmission and released when completed |
| LCD_A0 | PD0 | J4.5 | GPIO – OUT | The register select pin for the LCD. HIGH for reading/writing the display RAM, LOW for all other purposes |
| LCD_MOSI | PD3 | J4.6 | USART – OUT | The MASTER OUT pin for the SPI connection to the LCD |
| LCD_CLK | PD1 | J4.8 | USART – OUT | The CLOCK pulse output for the SPI connection to the LCD |
| LCD_LIGHT | PE4 | - | GPIO – OUT | The control pin for the backlight on the LCD. HIGH to switch on, LOW for off. Can use a PWM output for varying brightness |
| LCD_RST | PA3 | J3.4 | GPIO – OUT | The RESET pin for the LCD. Perform a hard reset pulling LOW |

## Switch Pins

Table 8 - Xmega Switch pin list

| Label | Xmega I/O | Xmega Connector | Signal type | Description |
| --- | --- | --- | --- | --- |
| SW0 | PE5 | - | GPIO – IN | Mechanical button, active LOW |
| SW1 | PF1 | - | GPIO – IN | Mechanical button, active LOW |
| SW2 | PF2 | - | GPIO – IN | Mechanical button, active LOW |

## Extra Pins

Table 9 - Xmega extras pin list

| Label | Xmega I/O | Xmega Connector | Signal type | Description |
| --- | --- | --- | --- | --- |
| SPARE_SW | PA5 | J2.6 | GPIO – OUT | A digital signal to control the spare analogue switch on board. LOW = open, HIGH = closed. |
| DEBUG_1 | PC5 | J1.6 | GPIO | Spare pin for debugging purposes. If DBG LED is fitted: active LOW |
| DEBUG_2 | PC7 | J1.8 | GPIO | Spare pin for debugging purposes. If DBG LED is fitted: active LOW |

# Test Points

Detailed in Table 10 is a list of the test points included on the board and what signals they are connected to. Locations of the test points can be found by name on the board or referring to the assembly layout in Appendix C.

Table 10 - Test point list

| Test Point | Description |
| --- | --- |
| MUX | Direct output of the multiplexer – preamplification |
| AMP | Amplified output signal – pre-DC bias removal + filtering |
| JACK | Main output signal to the jack plug |
| ENC_A | Quadrature signal A from rotary encoder |
| ENC_B | Quadrature signal B from rotary encoder |
| +3.3V | Positive 3.3V supply (from Xmega) |
| CS | Digipot chip select |
| CLK | Digipot clock signal |
| MOSI | Digipot data transmission line: DG_MOSI |
| MISO | Digipot data feedback receive line: DG_MISO |
| DBG1 | Spare debugging pin: DEBUG_1 |
| DBG2 | Spare debugging pin: DEBUG_2 |
| +16-24V | Main power in from plug or terminals |
| GND | Board ground |
| -12V | Negative 12V supply |
| +12V | Positive 12V supply |
| +5V | Positive 5V supply |

# Appendix

## Appendix A

Triangle Wave Generator Frequency Calculations

Starting with the equation for charging a capacitor

dVdt=iC

and substituting the saturation voltage of the op amp (as it is this voltage that drives the charging) with the resistance.

dVdt=VsatRC

The peak-to-peak voltage is defined as the difference between an upper and lower threshold. Assuming these thresholds are equal and opposite gives

Vpp=2∙RnRf∙Vsat=2HVsat

The time to complete half a period is found from the time taken to charge the full peak-to-peak

T2=VppdVdt=2HVsatVsatRC=2HRC

Therefore, full period and frequency are as shown

T=4HRC

f=1T=14HRC

## Appendix B

Sin Wave Generator Calculations

R1

Vin

C1

R2

C2

Vout

GND

GND

The divider network has a transfer function of

βω=Z2Z1+Z2

Where Z1=R1-jXC1, and Z2=R2||-jXC2

Z2=-jXC2∙R2-jXC2+R2

Z2=R21+R2-jXC2

With XC=1ωC,

Z1=R1-jωC1

Z2=R21+jωR2C2

Therefore,

βω=R21+jωR2C2R21+jωR2C2+R1-jωC1

βω=R2R21+C2C1+R1+jωR1R2C2-1ωC1

The least attenuated and therefore “selected” frequency is when there is no phase change and so can be found by setting the imaginary portion of the transfer function to 0.

ωR1R2C2-1ωC1=0

ωR1R2C2=1ωC1

ω2=1R1R2C1C2

ω=1R1R2C1C2

As ω=2πf

f=12πR1R2C1C2

The gain of the network is found from the magnitude of the transfer function. From above, we assume that the imaginary part is 0 and so the gain can be found from

βω=R2R21+C2C1+R1

Assuming that equal values of components are used

βω=RR1+CC+R

βω=R3R=13

This shows that the amplifier will attenuate the selected frequency by 13 and such extra gain of >3 is required for oscillation.

## Appendix C

Assembly Diagram

## Appendix D

3D Model

## Appendix E

Frequency from resistance of triangle oscillator core

Frequency from resistance of sin wave oscillator core