| Document No: | [Subject] | Issue: | [Status] | Effective date: | [Publish Date] |
| --- | --- | --- | --- | --- | --- |

| Document Title: |  |  |
| --- | --- | --- |
| [Title] |  |  |
| Summary / Scope: |  |  |
| Reason for Issue / Nature of Change: |  |  |
| Issue; Date; Details – reason for issue / nature of change. 01. 02. 03. |
| AUTHORISATION. Name:; Job title:; Signed:. Author:; Click to enter text. Reviewer:; Click to enter text. Approver / Owner:; Click to enter text. Authorisation details required irrespective of document approval process applied.; Electronic signatures required only where SharePoint approval workflows are not being used.. |

Copyright Statement

© 2026 Renishaw plc

This document is the property of Renishaw plc and the information contained herein is confidential.  The contents of the document must not be reproduced or disclosed wholly or in part, or used for purposes other than that for which it has been supplied, without the prior written permission of Renishaw plc, or, if it has been furnished under a contract with another party, as expressly authorised under that contract.

Table of Contents

1	Introduction	3

2	Test Items	3

3	Setup	3

4	Test Plans	3

4.1	PCB power	3

4.2	IC power	3

4.3	Generated signal amplitudes	4

4.4	MUX Signal	4

4.5	Output Signal Amplitude	4

4.6	Output signal Integrity	4

4.7	Output Signal Frequency	4

4.8	PWM Duty Cycle	5

# Introduction

# Test Items

- PCB power

- IC power

- Generated signal amplitudes

- MUX signal

- Output signal amplitude

- Output signal frequency

- Output signal integrity

- PWM duty signal

# Setup

- Connect SK4 with wires to an external power supply with +12V, -12V and Ground

- Connect PL1, PL2 & PL4 to 100kΩ variable resistances

- Put all switches on SW2 into the OFF position

- Connect PL3 to a 10kΩ variable resistance and set to just over 50% (roughly 5-6kΩ)

- Set variable resistances on PL2, PL3 and PL4 to the same (all fully CW or fully CCW)

- Turn RV2 through 50% of its rotation

# Test Plans

## PCB power

- Turn on the power supply and probe the pins of SK4 for the relevant power levels to ensure correct connectivity

- Probe TP2, TP3 and TP4 and compare to the pins on SK4

| Testpoint | Expected Voltage |
| --- | --- |
| SK4.1 | 0V |
| SK4.2 | +12V |
| SK4.3 | -12V |
| SK4.4 | 0V |
| TP2 | +12V |
| TP3 | -12V |
| TP4 | 0V |

## IC power

Probe the IC power pins to ensure they are receiving correct levels.

| Testpoint | Expected Voltage |
| --- | --- |
| IC2.3 | 0V |
| IC2.4 | -12V |
| IC2.8 | +12V |
| IC3.4 | +12V |
| IC3.5 | 0V |
| IC3.8 | +12V |
| IC4.4 | -12V |
| IC4.6 | 0V |
| IC4.8 | +12V |
| IC5.4 | -12V |
| IC5.5 | 0V |
| IC5.8 | +12V |

## Generated signal amplitudes

Probe TP6, TP7  TP8 and check amplitude

| Testpoint | Expected Amplitude |
| --- | --- |
| TP6 | 10V |
| TP7 | 10V |
| TP8 | 10V |

## MUX Signal

Probe TP1 and compare according to the following configurations

| SW1.1 | SW1.2 | Expected Signal |
| --- | --- | --- |
| 0 | 0 | PWM |
| 0 | 1 | Triangle |
| 1 | 0 | Sin |
| 1 | 1 | Ground |

Amplitude is expected to be 10V for each case too.

## Output Signal Amplitude

- Probe TP5 while varying RV1 through its full range and compare to expected graph

- Repeat on each signal setting by changing SW1.1 and SW1.2

## Output signal Integrity

During the next testing period: while sweeping through the values, analyse the shape of the waves visually - looking for symmetry in rise and fall times and shapes.

## Output Signal Frequency

- Return RV1 to its fully CW position

- Turn switch SW1.1 OFF and switch SW1.2 ON (Triangle setting)

- Switch SW2.3 ON

- Probe TP5 and measure frequency while varying the resistance across PL1

- Switch SW2.3 OFF and repeat step 4

- Turn switch SW1.1 ON and switch SW1.2 OFF (Sinusoidal setting)

- Probe TP5 and measure frequency while varying the resistances equally across both PL2 and PL4. (Resistance of PL3 may have to be varied slightly to allow oscillation)

- Switch SW2.3 ON and repeat step 7

## PWM Duty Cycle

- Turn both switches SW1.1 and SW1.2 OFF (PWM setting)

- Probe TP5 and inspect the duty cycle while varying RV2 across all positions